Group 3

IV-Quasar

Candle Making

Ingredients

- Paraffin wax ( consists 7 parts of the total candle )

- Dye ( for making different colors )

- 2 parts microwax

- 1 part crystal wax

- Essential oil

Materials

- Containers for mixing colors and melting wax

- Molds (Plaster of Paris of aluminium)

- Double boiler

- Water heater

- Wick (made of bleached cotton yarn)

- Ladle (for pouring the liquid wax into the mold)

- Basin

Procedure

- Cut paraffin wax into pieces. Put in container and heat in double boiler until wax is completely melted. (This can also be done by utilizing a cooking stove and heating pieces of wax in a cooking pan)

- Add dye to molten paraffin wax.

- Add microwax to molten paraffin wax and mix until microwax is completely molten.

- Heat crystal wax in a separate container and add it to paraffin wax-microwax mixture.

- Add 1 teaspoon of essential oil per liter of wax solution to make candles smooth and hard. Stearic acid may be used instead of essential oils.

- Prepare molds (in two parts) on which to pour wax by taking out clamps to separate mold.

- Apply oil into mold cavities to separate candle easily.

- Insert wick into molds and bring both parts of mold together and clip.

- Using a big ladle, pour molten wax in mold, filling it to the mouth.

- Put mold in a basin of cold water for a few minutes to solidify wax.

- Take out mold from water, open, and remove candles.

- Cut off excess wick.